Osterquill Toxicology Inc

# Standard Operating Procedure OQ-QA-014

Control of Individual-Animal Cards

Revision 03

Effective date: 02/20/2024

Supersedes: Revision 02

# 1.0 Purpose

This procedure defines how individual-animal cards are issued, completed, corrected, reconciled and archived so that every observation in an in vivo investigation can be reconstructed.

# 2.0 Scope

Applies to all in vivo investigations at Osterquill. GLP studies follow every section. Exploratory investigations follow sections 5.1 to 5.5; QA audit of exploratory records is for data integrity only and does not constitute a GLP compliance statement.

# 3.0 Responsibilities

Study Director: approves card templates and reviews control validity

In vivo technicians: make contemporaneous entries

Quality Assurance: audits cards and releases them to the archive

# 4.0 Definitions

Animal card: the primary record of observations, body weights and dosing for one animal

Dose lot: a prepared formulation batch identified on the dose record

Control validity: confirmation that vehicle and reference groups behaved within historical expectations

# 5.0 Procedure

## 5.1 Card issue

Cards are printed from the approved template for each investigation, pre-numbered and logged in the card issue register before animal allocation.

## 5.2 Entries

Record observations at the time they are made, in indelible ink

Each entry is initialed and dated

Do not leave blank lines between entries; strike through unused space at the end of each day

## 5.3 Corrections

Strike through the original entry with a single line so it remains legible, write the correction alongside, and initial, date and give the reason. Correction fluid and overwriting are not permitted.

## 5.4 Dose-lot reconciliation

At the end of each dosing day the dose lot on every card is reconciled against the dose preparation record. Discrepancies are reported to the Study Director the same day.

## 5.5 Welfare escalation

Any observation meeting a humane endpoint criterion is reported immediately to the attending veterinarian and the Study Director, and the action taken is recorded on the card.

## 5.6 Control validity review

Before the report is drafted, the Study Director reviews vehicle and reference group data against historical control ranges and documents the conclusion.

## 5.7 Archive release

QA audits the completed card set, confirms reconciliation and correction compliance, and releases the cards to the archive with an archive index entry. Cards are retained for at least 10 years.

# 6.0 Revision history

| Rev | Date | Change |
| --- | --- | --- |
| 01 | 04/14/2014 | New SOP |
| 02 | 09/08/2019 | Added dose-lot reconciliation |
| 03 | 02/20/2024 | Separated GLP and exploratory requirements; added welfare escalation |

| Name | Role | Signature | Date |
| --- | --- | --- | --- |
| Lucia Moretti | GLP Quality Assurance and Archive Lead | Moretti | 02/06/2024 |
| Naomi Takamori | Head of Toxicology | Takamori | 02/06/2024 |